EXPERIMENT  17

AIM:

PERFORMANCE EVALUATION OF DIGITAL COMMUNICATION SYSTEMS USING EYE DIAGRAM ANALYSIS UNDER ADDITIVE WHITE GAUSSIAN NOISE (AWGN) USING MATLAB

SOFTWARE REQUIRED:

MATLAB Simulation

PROGRAM:

Clc;

Clear;

Close all;

N = 1000;

Sps = 20;

Data = randi([0 1],1,N);

X = 2*data-1;

Tx = repelem(x,sps);

% Add AWGN

R1 = tx + 0.2*randn(size(tx));

R2 = tx + 0.8*randn(size(tx));

Figure;

Subplot(3,1,1);

Plot(tx(1:400));

Title(‘Transmitted BPSK Signal’);

Grid on;

Subplot(3,1,2);

Eyediagram(r1(1:4000),2*sps);

Title(‘Eye Diagram with Low Noise’);

Subplot(3,1,3);

Eyediagram(r2(1:4000),2*sps);

Title(‘Eye Diagram with High Noise’);

GRAPH:

OUTPUT :

RESULT:

The eye diagram was successfully analyzed using MATLAB under AWGN. Increasing noise reduced the eye opening and degraded digital communication performance.